# ASCOM ALPACA on the ZWO Seestar S50:

# A Technical Reference

March 2026

## Abstract

The ZWO Seestar S50 is a compact, fully automated smart telescope that originally exposed a proprietary JSON-RPC 2.0 control interface over TCP. Beginning with firmware version 3.0, ZWO introduced cryptographic authentication on this interface, effectively locking out all third-party control software. Simultaneously, ZWO added a native ASCOM ALPACA HTTP REST server on TCP port 32323, providing a standards-based alternative for external motor control. This document catalogues the findings of systematic protocol analysis and empirical testing of both interfaces on firmware 3.1.1, and characterizes the capabilities and limitations of the ALPACA implementation on the Seestar S50.

## 1. Background: The JSON-RPC Interface (Port 4700)

### 1.1 Protocol Overview

The Seestar S50 exposes a JSON-RPC 2.0 interface over a persistent TCP connection on port 4700. Messages are newline-delimited (\r\n), with one complete JSON object per line. The interface is bidirectional: the client sends commands and receives both solicited responses (keyed by message ID) and unsolicited push events.

A dedicated reader thread is required on the client side. The scope pushes events asynchronously — PiStatus (temperature, battery, charger state), ScopeTrack (motion state), FocuserMove, RecordingStart/RecordingStop, viewing mode transitions, and Client (master control status). Without a reader thread continuously draining the socket, responses are lost, push events queue silently, and all synchronous commands eventually timeout.

### 1.2 Connection Handshake

The connection sequence is non-trivial and must occur in the correct order:

- UDP broadcast — A scan_iscope JSON-RPC message is broadcast to 255.255.255.255:4720 before the TCP connection. This satisfies the scope’s “guest mode” handshake; without it, the scope treats the TCP client as a ghost and silently ignores commands.

- TCP connect — Standard socket connection to port 4700.

- Initialization sequence — A series of fire-and-forget commands:

  - set_user_location (latitude, longitude, force=true) — syncs observer coordinates.

  - pi_set_time (year, month, day, hour, minute, second, time_zone="UTC") — syncs the real-time clock.

  - pi_is_verified — session verification.

  - set_setting {"master_cli": true} — claims master control.

  - set_setting {"cli_name": "<identifier>"} — client identification.

- Heartbeat — A keep-alive pi_is_verified message must be sent every ~3 seconds to prevent the scope from dropping the connection.

### 1.3 Master Control

The Seestar firmware supports only one master client at a time. Without master status, the scope accepts motor commands syntactically but silently discards them — no error is returned. The scope announces master status via a push Client event containing {"is_master": true|false}. If the Seestar iPhone app is open (even in background), it typically holds master, and the third-party client must reclaim it by re-sending set_setting {"master_cli": true}.

### 1.4 The verify Parameter

Firmware versions introduced a verify parameter with version-dependent injection rules:

| Firmware | Parameter format | Rule |
| --- | --- | --- |
| <2706, dict params | Dict | Inject "verify": true into the params dict |
| >=2706, dict params | Dict | Omit verify entirely (error code 109 if present) |
| Any, list params | List | Append the string "verify" to the list |
| Any, no params | — | Add top-level "verify": true |

This is a firmware handshake mechanism, not cryptographic authentication in the traditional sense.

### 1.5 Command Taxonomy

The JSON-RPC interface exposes commands across several domains:

Viewing modes (fire-and-forget; confirmed by push events): - iscope_start_view — Enter solar (mode: "sun"), lunar (mode: "moon"), scenery (mode: "scenery"), or star (mode: "star" with target_ra_dec) mode. - iscope_stop_view — Exit current viewing mode.

Recording (fire-and-forget; confirmed by RecordingStart/RecordingStop events): - start_record_avi {"raw": false} — Begin MP4 recording. - stop_record_avi — End recording.

Motor control (synchronous or fire-and-forget): - scope_speed_move — Nudge at a given speed (0–8000 scale), angle (with +180° firmware offset), and duration. - scope_open — Open (unfold) the telescope arm. - scope_park — Park (fold) the arm.

Camera and optics (synchronous): - start_auto_focuse — Autofocus (note: typo in firmware command name). - move_focuser — Manual focus step. - set_control_value ["gain", N] — Set sensor gain. - set_setting {"exp_ms": ...} — Set exposure time. - set_setting {"stack_lenhance": bool} — Light pollution filter toggle. - pi_output_set2 {"heater": ...} — Dew heater control.

Telemetry queries (synchronous, require reader thread): - scope_get_horiz_coord — Alt/Az readout. - scope_get_equ_coord — RA/Dec readout. - get_view_state — Current viewing mode. - get_device_state — Device state summary.

### 1.6 Firmware 3.0+ Lockout

Testing on firmware 3.1.1 (Seestar App v3.0+) revealed the following behavioral changes:

| Observation | Evidence |
| --- | --- |
| TCP port 4700 still accepts connections | Socket stays ESTABLISHED |
| Scope sends PiStatus push events (temperature, battery) | Received by reader thread |
| All query/response commands timeout (5–8 s) | Tested: scope_get_horiz_coord, get_view_state, scope_get_equ_coord, get_device_state, pi_is_verified, get_setting |
| Scope sends zero responses to any command | Raw socket reader receives nothing back |
| scope_speed_move produces no physical movement | Tested at speed=4000, dur=10 s |
| UDP port 4720 accepts broadcast but sends no reply | No connection refused; no response |
| iscope_start_view mode=sun appears functional | Scope tracks sun on startup; unclear if command or default behavior |

The root cause, confirmed by the maintainer of the seestar_alp project (GitHub issue #697), is that ZWO intentionally added cryptographic authentication to the port 4700 interface. Third-party applications can connect and receive temperature telemetry, but all motor commands and query responses are silently suppressed.

What still works on port 4700 (firmware 3.0+): - TCP connection and push event reception (PiStatus, ScopeTrack, FocuserMove, etc.) - Viewing mode commands (iscope_start_view, iscope_stop_view) — fire-and-forget, confirmed via push events - Recording commands (start_record_avi, stop_record_avi) — fire-and-forget, confirmed via events - Camera settings (gain, exposure, focus, dew heater) — fire-and-forget

What no longer works on port 4700: - All query/response commands (position readout, state queries) - Motor movement (scope_speed_move, scope_open, scope_park) - Any command requiring a synchronous response

## 2. The ALPACA Interface (Port 32323)

### 2.1 Protocol Overview

ASCOM ALPACA is a platform-independent REST/JSON standard for astronomy device control, maintained by the ASCOM Initiative. The Seestar S50 firmware 3.0+ ships a native ALPACA server that exposes the ASCOM ITelescope interface over HTTP on TCP port 32323.

The protocol is stateless HTTP: properties are read via GET, commands are executed via PUT with application/x-www-form-urlencoded bodies. Every request includes ClientID and ClientTransactionID parameters; every response includes these plus a ServerTransactionID, an ErrorNumber (0 = success), and an ErrorMessage.

No authentication or encryption is used. The server binds to all interfaces.

### 2.2 Discovery

ALPACA defines a UDP discovery protocol. A client broadcasts the ASCII string alpacadiscovery1 to 255.255.255.255:32227. ALPACA-capable devices reply with a JSON payload containing at minimum the AlpacaPort field:

→ UDP broadcast to 255.255.255.255:32227: "alpacadiscovery1"
← Reply from 192.168.x.x:  {"AlpacaPort": 32323}

This is distinct from the proprietary ALP discovery on port 4720.

### 2.3 Management Endpoints

Three management endpoints exist outside the device API namespace:

| Endpoint | Returns |
| --- | --- |
| GET /management/v1/description | Server name, manufacturer, version |
| GET /management/v1/configureddevices | Array of device type/number/name/uniqueid entries |
| GET /management/apiversions | Supported API version integers |

On the Seestar, /management/v1/configureddevices returns a single telescope device at index 0.

### 2.4 API Structure

All telescope endpoints live under:

http://{host}:32323/api/v1/telescope/0/{endpoint}

GET requests pass parameters as query strings:

GET /api/v1/telescope/0/canmoveaxis?Axis=0&ClientID=1&ClientTransactionID=5

PUT requests pass parameters as form-encoded bodies:

PUT /api/v1/telescope/0/moveaxis
Content-Type: application/x-www-form-urlencoded

Axis=0&Rate=1.0&ClientID=1&ClientTransactionID=6

Response format (all endpoints):

{
  "Value": true,
  "ErrorNumber": 0,
  "ErrorMessage": "",
  "ClientTransactionID": 5,
  "ServerTransactionID": 42
}

### 2.5 Capability Profile

The Seestar S50 reports the following capabilities via ALPACA property reads:

| Capability | Value | Implication |
| --- | --- | --- |
| canslew | true | Synchronous RA/Dec GoTo supported |
| canslewasync | true | Asynchronous RA/Dec GoTo supported |
| canslewaltaz | false | No native Alt/Az GoTo — clients must convert to RA/Dec |
| canslewaltazasync | false | Same limitation |
| canmoveaxis | true | Rate-based axis nudge supported (both axes) |
| canpark | true | Park/unpark (arm fold/unfold) supported |
| canpulseguide | false | No autoguider pulse guide |
| cansettracking | true | Sidereal tracking toggle supported |
| interfaceversion | 3 | ASCOM ALPACA interface version 3 |

The canslewaltaz=false limitation is significant. To slew to a known altitude and azimuth, the client must perform the spherical trigonometric conversion to equatorial coordinates using the observer’s latitude, longitude, and the current Greenwich Apparent Sidereal Time.

### 2.6 Confirmed Working Operations

Tested on firmware 3.1.1 with the scope in station mode (connected to home WiFi):

Position readout: - GET rightascension — RA in hours [0, 24) - GET declination — Dec in degrees [-90, +90] - GET altitude — Alt in degrees - GET azimuth — Az in degrees [0, 360) - GET siderealtime — Local Sidereal Time in hours - GET utcdate — Scope clock as ISO 8601 string

Motor control: - PUT moveaxis with Axis=0, Rate=1.0 — Primary axis (RA/Az) nudge at 1 deg/s. Rate=0 stops. Both axes (0 and 1) confirmed independently. Motor continues at constant rate until a Rate=0 stop command is issued. - PUT slewtocoordinatesasync with RightAscension=<hours>, Declination=<degrees> — Asynchronous GoTo. Returns immediately; poll GET slewing to detect completion. Confirmed: scope physically moves to target coordinates. - PUT abortslew — Halts any in-progress slew.

Tracking: - PUT tracking with Tracking=true|false — Enables or disables sidereal tracking. Confirmed working. - GET tracking — Returns current tracking state.

Park/unpark: - PUT park — Parks (folds arm). Confirmed. - PUT unpark — Unparks (opens arm). Confirmed. - GET atpark — Returns boolean park state.

Connection: - PUT connected with Connected=true — Opens ALPACA session. No master control negotiation required (unlike JSON-RPC). - PUT connected with Connected=false — Closes session.

### 2.7 Operations Not Available via ALPACA

The Seestar’s ALPACA server exposes only the standard ASCOM ITelescope interface. The following Seestar-specific functions have no ALPACA equivalent:

| Function | Notes |
| --- | --- |
| Solar/lunar/scenery viewing modes | Proprietary; use JSON-RPC iscope_start_view |
| Video recording (AVI/MP4) | No ASCOM ICamera device exposed |
| Photo capture | No camera device |
| Sensor gain and exposure control | No camera device |
| Autofocus | No ASCOM IFocuser device exposed |
| Manual focus stepping | No focuser device |
| Dew heater control | No ASCOM ISwitch device exposed |
| Light pollution filter toggle | Proprietary setting |
| RTSP live preview stream | Proprietary; accessible at rtsp://{host}:4554/stream |
| File management (album listing, download) | Proprietary HTTP endpoints on scope |
| Device telemetry (CPU temp, battery, charger) | Proprietary push events on port 4700 |

The ASCOM standard defines an ITelescopeV4.Action() method for device-specific extensions, but testing has not confirmed whether ZWO exposes any proprietary actions through this endpoint.

### 2.8 Alt/Az to RA/Dec Conversion

Since canslewaltaz is false, clients must convert altitude/azimuth to right ascension/declination before issuing a GoTo. The conversion uses standard spherical trigonometry:

sin(Dec) = sin(Alt) * sin(Lat) + cos(Alt) * cos(Lat) * cos(Az)
Dec = arcsin(sin(Dec))

cos(HA) = (sin(Alt) - sin(Lat) * sin(Dec)) / (cos(Lat) * cos(Dec))
HA = arccos(cos(HA))
if sin(Az) > 0:  HA = 2π - HA

LST = GAST + Longitude/15
RA = LST - HA * 12/π   (mod 24)

Where Lat is the observer’s latitude, GAST is Greenwich Apparent Sidereal Time, HA is the hour angle, and LST is the local sidereal time. All intermediate calculations are in radians; RA is output in hours, Dec in degrees.

## 3. Hybrid Architecture

Given the complementary capabilities of the two interfaces, a practical control architecture for the Seestar S50 on firmware 3.0+ uses both simultaneously:

| Domain | Interface | Port | Protocol |
| --- | --- | --- | --- |
| Motor control (GoTo, nudge, stop) | ALPACA | 32323 | HTTP REST |
| Tracking enable/disable | ALPACA | 32323 | HTTP REST |
| Park / unpark | ALPACA | 32323 | HTTP REST |
| Position readout (RA/Dec/Alt/Az) | ALPACA | 32323 | HTTP REST |
| Viewing modes (solar, lunar, scenery) | JSON-RPC | 4700 | TCP |
| Recording start/stop | JSON-RPC | 4700 | TCP |
| Camera settings (gain, exposure, focus) | JSON-RPC | 4700 | TCP |
| Device telemetry (temp, battery) | JSON-RPC | 4700 | TCP push events |
| RTSP live preview | Proprietary | 4554 | RTSP/TCP |
| Discovery | Both | 4720 (ALP) / 32227 (ALPACA) | UDP broadcast |

The JSON-RPC connection must be maintained for its push event stream (the only source of recording state, viewing mode changes, and device telemetry) and for issuing viewing mode and recording commands. The ALPACA connection handles all motor control — the only domain where JSON-RPC is now locked out.

## 4. Operational Requirements and Constraints

### 4.1 Network Mode

ALPACA only functions when the Seestar is in station mode — connected to a local WiFi network. In AP mode (the scope’s own hotspot), port 32323 is not exposed. Station mode is configured through the Seestar iPhone/Android app.

### 4.2 Arm State

The telescope arm must be opened (unparked) before motor commands will execute. This can be done via the unpark ALPACA command or through the Seestar app. Attempting moveaxis or slewtocoordinatesasync while parked returns an ALPACA error.

### 4.3 No Authentication

Neither interface employs network-level authentication. The ALPACA server binds to all interfaces on port 32323 without TLS or access control. Any device on the same network can issue commands. Deployments should restrict network access accordingly.

### 4.4 Polling vs Push

ALPACA is strictly request/response — there is no push event mechanism. Clients must poll for state changes (position, slewing status, tracking state). A 2–3 second polling interval is practical. JSON-RPC, by contrast, pushes events asynchronously and requires a persistent reader thread.

## 5. Summary of Protocol Differences

| Aspect | JSON-RPC (Port 4700) | ALPACA (Port 32323) |
| --- | --- | --- |
| Transport | Persistent TCP socket | Stateless HTTP |
| Encoding | JSON-RPC 2.0, newline-delimited | REST/JSON, form-encoded PUT bodies |
| Discovery | UDP broadcast, port 4720 | UDP broadcast, port 32227 |
| Authentication | verify param (handshake, not crypto) | None |
| Session management | Master control claim required | connected=true sufficient |
| Heartbeat | Required (3 s pi_is_verified) | Not required |
| Push events | Yes (PiStatus, ScopeTrack, etc.) | No — polling only |
| Motor control (fw 3.0+) | Silently dropped | Fully functional |
| Viewing modes | Functional | Not exposed |
| Recording | Functional | Not exposed |
| Camera/focuser/heater | Functional | Not exposed |
| Position readout (fw 3.0+) | Timeout (queries blocked) | Fully functional |

## References

- ASCOM ALPACA API specification — https://ascom-standards.org/api/

- seestar_alp — Open-source Seestar controller: https://github.com/smart-underworld/seestar_alp

  - Key source: device/seestar_device.py — guest_mode_init(), transform_message_for_verify(), is_client_master(), move_scope()

  - Issue #697: Confirmation of firmware 3.0+ lockout

- ZWO Seestar S50 product page — https://www.zwoastro.com/seestar/